COURSE OUTCOME COVERED

CO: 2 - Analyse LTE and 5G wireless network architectures, including carrier aggregation, MIMO techniques, beamforming strategies, and service-based core network functional entities. (BL4)

Assessment Tool Number 		: CO2_AT2

Assessment Tool Weightage	: 35%

Assessment Tool			: Simulation Task

Duration				: Self Learning

Marks					: 100 Marks

Type of Assessment Conduction  : At Home

Evaluation Type			: Online Evaluation(GitHub)

Result Generation	: Students successfully simulate the assigned wireless communication system, analyze the obtained results, and interpret the performance based on relevant technical parameters.

Topic Covered	: LTE/LTE-Advanced architecture, protocol stack, carrier aggregation, higher-order MIMO, beamforming, and beamsteering techniques.

5G NR air interface, gNodeB and RAN architecture, RRC procedures, 5G Core, Service-Based Architecture, network slicing, AMF, SMF, UPF, and control/user plane separation.

Conduction of Assessment Procedure:

- Assessment will be carried out through rubric-based evaluation, where students complete the assigned task.

- Simulation will be done in MATLAB/Simulink for the given specification and interpret the results.

- TWO questions will be assigned to students each carrying 10 marks and they are required to collaboratively discuss the given problem and present their responses with clear justification and submit the document in GitHub Platform.

1.Simulation of analog beamforming performance under phase shift variation, beam angle, antenna count, gain, and sidelobe level (BL4)

2.Simulation of 5G core traffic routing under user density, mobility, latency, throughput,                  and resource utilization (BL4)

Question 01:

Program:

clc;

clear;

close all;

%% 1. Parameters Setup

c = 3e8; % Speed of light (m/s)

fc = 28e9; % Carrier frequency (28 GHz - 5G mmWave)

lambda = c / fc; % Wavelength

d = lambda / 2; % Half-wavelength antenna spacing

N = 8; % Default number of antenna elements

theta = -90:0.1:90; % Angular range in degrees

theta_rad = deg2rad(theta);

%% 2. Analog Beamforming Pattern for Different Steering Angles

beamAngles = [-30, 0, 30]; % Target beam directions in degrees

figure('Name', 'Analog Beamforming Steering Angles');

for k = 1:length(beamAngles)

theta0 = beamAngles(k);

theta0_rad = deg2rad(theta0);

% Progressive phase shift required for beam steering at theta0

beta = -2 * pi * (d / lambda) * sin(theta0_rad);

% Array Factor calculation

psi = 2 * pi * (d / lambda) * sin(theta_rad) + beta;

AF = abs(sin(N * psi / 2) ./ (N * sin(psi / 2)));

AF(isnan(AF)) = 1; % Handle 0/0 limit (main lobe peak)

% Plot normalized radiation pattern in dB

subplot(length(beamAngles), 1, k);

plot(theta, 20*log10(AF), 'LineWidth', 1.8);

grid on;

ylim([-40, 2]);

xlim([-90, 90]);

xlabel('Angle (Degrees)');

ylabel('Normalized Gain (dB)');

title(['Beam Steering Angle (\theta_0) = ', num2str(theta0), '°']);

end

%% 3. Sidelobe Level (SLL) Analysis & Phase Shift Variation Effect

% Simulating phase error/variation (e.g., non-ideal phase shifters)

phase_error_std = 10; % Standard deviation of phase error in degrees

% Ideal steering at 0 degrees

beta_ideal = zeros(1, N);

% Non-ideal steering with random phase shift variation

rng(1); % For reproducibility

phase_noise = deg2rad(normrnd(0, phase_error_std, [1, N]));

beta_noisy = beta_ideal + phase_noise;

% Calculating Array Factor with phase shift variations

AF_ideal = zeros(size(theta));

AF_noisy = zeros(size(theta));

for i = 1:length(theta)

% Steering vector for element positions [0, 1, ..., N-1]

n = 0:N-1;

spatial_phase = 2 * pi * (d / lambda) * n * sin(theta_rad(i));

AF_ideal(i) = abs(sum(exp(1i * (spatial_phase + beta_ideal))));

AF_noisy(i) = abs(sum(exp(1i * (spatial_phase + beta_noisy))));

end

AF_ideal = AF_ideal / max(AF_ideal);

AF_noisy = AF_noisy / max(AF_noisy);

figure('Name', 'Phase Shift Variation & Sidelobe Level');

plot(theta, 20*log10(AF_ideal), 'b', 'LineWidth', 1.8, 'DisplayName', 'Ideal Phase');

hold on;

plot(theta, 20*log10(AF_noisy), 'r--', 'LineWidth', 1.5, 'DisplayName', 'Phase Error (\sigma = 10°)');

grid on;

ylim([-40, 2]);

xlim([-90, 90]);

xlabel('Angle (Degrees)');

ylabel('Normalized Gain (dB)');

title('Impact of Phase Shift Variation on Sidelobe Level (SLL)');

legend('Location', 'northeast');

%% 4. Array Gain vs. Antenna Count (N)

antenna_counts = [4, 8, 16, 32, 64];

array_gain_dB = 10 * log10(antenna_counts); % Gain G = 10*log10(N)

figure('Name', 'Array Gain vs Antenna Count');

plot(antenna_counts, array_gain_dB, '-o', 'LineWidth', 2, 'MarkerSize', 8, 'MarkerFaceColor', 'r');

grid on;

xlabel('Number of Antenna Elements (N)');

ylabel('Array Gain (dB)');

title('Array Gain vs. Antenna Count');

xticks(antenna_counts);

Outputs:

Question 2:

Program :

clc;

clear;

close all;

%% 5G Core Traffic Routing Simulation

% User Density

userDensity = 100:100:1000;

% Mobility (km/h)

mobility = [10 20 40 60 80 100 120 140 160 180];

% Performance Parameters

latency = 5 + 0.015*userDensity; % ms

throughput = 1200 - 0.6*userDensity; % Mbps

resourceUtilization = 30 + 0.06*userDensity; % %

packetLoss = 0.5 + 0.005*userDensity; % %

%% Figure 1: User Density vs Latency

figure;

plot(userDensity, latency,'-o','LineWidth',2);

grid on;

xlabel('User Density');

ylabel('Latency (ms)');

title('5G Core Routing: User Density vs Latency');

%% Figure 2: User Density vs Throughput

figure;

plot(userDensity, throughput,'-s','LineWidth',2);

grid on;

xlabel('User Density');

ylabel('Throughput (Mbps)');

title('5G Core Routing: User Density vs Throughput');

%% Figure 3: User Density vs Resource Utilization

figure;

plot(userDensity, resourceUtilization,'-^','LineWidth',2);

grid on;

xlabel('User Density');

ylabel('Resource Utilization (%)');

title('5G Core Routing: User Density vs Resource Utilization');

%% Figure 4: User Density vs Packet Loss

figure;

plot(userDensity, packetLoss,'-d','LineWidth',2);

grid on;

xlabel('User Density');

ylabel('Packet Loss (%)');

title('5G Core Routing: User Density vs Packet Loss');

%% Figure 5: Mobility vs Throughput

throughputMobility = 1100 - 2*mobility;

figure;

plot(mobility, throughputMobility,'-*','LineWidth',2);

grid on;

xlabel('Mobility (km/h)');

ylabel('Throughput (Mbps)');

title('Mobility vs Throughput');

%% Display Results

disp('-------------------------------');

disp('5G Core Traffic Routing Results');

disp('-------------------------------');

Result = table(userDensity', latency', throughput', ...

resourceUtilization', packetLoss', ...

'VariableNames', {'UserDensity','Latency_ms','Throughput_Mbps',...

'ResourceUtilization_percent','PacketLoss_percent'});

disp(Result);

Outputs :